# ELDERLY  FALL & HEALTH MONITORING DEVICE

PRODUCT RE-ENGINEERING AND INNOVATION (ST 251)

A PROJECT REPORT

Submitted by

AYUSHI(UE245018)

DHWANI(UE245028)

In partial fulfillment for the award of the degree

of

BACHELOR OF ENGINEERING IN

ELECTRONICS AND COMMUNICATION

ENGINEERING (ECE)

UNIVERSITY INSTITUTE OF ENGINEERING

AND TECHNOLOGY SECTOR 25, CHANDIGARH

PANJAB UNIVERSITY, CHANDIGARH

160014

ABSTRACT

The increasing elderly population poses significant challenges to healthcare systems, especially in ensuring

their safety and well-being in daily life. One of the most critical issues among the elderly is accidental falls, which can lead to serious injuries, long-term disability, or even death if not addressed promptly. This project proposes the design and development of an Elderly Fall Detection and Health Monitoring Device, which aims to provide real-time fall detection and basic vital sign monitoring using a compact, wearable system. The device utilizes an accelerometer and gyroscope to detect unusual body movements and orientations indicative of a fall. In addition, sensors such as heart rate monitors and temperature sensors track key health parameters. Once a fall or abnormal reading is detected, the system immediately sends alerts to caregivers or emergency services via a mobile application or SMS using IoT-based communication. This device empowers families and healthcare providers with timely information, enabling faster response and improving the quality of care for the elderly. The system is designed to be low-cost, easy to wear, and reliable, making it a practical solution for enhancing senior safety and promoting independent living.

| CHAPTER NO. | TITLE | PAGE NO. |
| --- | --- | --- |
|  | ABSTRACT | i |
| 1. | INTRODUCTION - GENERAL - MOTIVATION - OBJECTIVES | ii 1 1 2 |
| 2. | WORKING PRINCIPLE & METHODOLOGY 2.1 BLOCK DIAGRAM 2.2 COMPONENTS USED AND COST 2.3 CIRCUIT DIAGRAM 2.4 WORKING EXPLAINATION | iii 1 2 3 3 |
| 3. | CODE DESCRIPTION 3.1 ARDUINO CODE | iv 1 |
| 4. | RESULTS & DISCUSSION 4.1 PROTOTYPE | V 1 |
| 5. | CONCLUSION,APPLICATIONS & FUTURE SCOPE | vi |
| 6. | REFERENCES | vii |
| 7. | APPENDICES | viii |

TABLE OF CONTENTS

CHAPTER 1. INTRODUCTION

  - GENERAL

As people age, their risk of falling and experiencing health-related issues increases. Falls are a leading cause of injury among the elderly and often go unnoticed when they live alone. To address this problem, an Elderly Fall and Health Monitoring Device is designed to automatically detect falls and monitor vital signs like heart rate and body temperature. This wearable device uses motion sensors and health sensors to track the user’s activity and physical condition. In case of a fall or abnormal health reading, it instantly sends alerts to family members or healthcare providers. This system helps ensure quick assistance, promotes independent living, and improves the safety and well-being of elderly individuals.

  - MOTIVATION

With the rise in the elderly population, ensuring their safety and health—especially when they live alone—has become a major concern. Falls are common and can be life-threatening if help is delayed. This motivated the idea of developing a smart, wearable device that can detect falls instantly and monitor vital health signs, allowing timely response and reducing the risk of serious injuries or complications. The goal is to support independent living while giving peace of mind to both the elderly and their caregivers

  - OBJECTIVES

- Fall Detection

- Health Monitoring

- Instant Alert System

Promote Independent Living

Chapter 2. WORKING PRINCIPLE & METHODOLOGY

2.1 BLOCK DIAGRAM

[Accelerometer/Gyroscope] [Heart Rate Sensor] [Temp Sensor]

| | |

+-----------+--------------+-----------+------------+

|

[Microcontroller]

|

+----------------+----------------+

| |

[GSM Module] [Buzzer Alert]

| |

[SMS to Caregiver] [Sound Alarm]

Fig 2.1

Block diagram of circuit

2.2  Components used and Cost

Items                                                                                    Cost

- Aurdino Nano/ ESP32                                                      ₹370.00

- ADXL345(Acclerator)                                                     ₹220.00

- MAX30102(Pulse Sensor)                                               ₹200.00

- LM35(Temperature Sensor)                                             ₹45.00

- ESP8266                                                                           ₹350.00

- Multimeter                                                                        ₹175.00

- Soldering Iron                                                                   ₹135.00

- Soldering Wire                                                                  ₹15.00

- Soldering Flux                                                                  ₹15.00

- Connecting Wires                                                             ₹ 20.00

- Breadboard                                                                       ₹85.00

- OLED Display                                                                  ₹100.00

- Jumper Wires                                                                    ₹60.00

TOTAL COST: ₹1790

2.3  CIRCUIT DIAGRAM

Fig 2.3

Circuit diagram consisting of components

2.4  WORKING EXPLAINATION

Accelerometer/Gyroscope (ADXL345):

Detects sudden movement or orientation changes to identify possible falls.

Heart Rate Sensor (e.g., MAX30100):

Measures the user's pulse by detecting changes in blood flow through light absorption.

Temperature Sensor (e.g., LM35):

Monitors body temperature and sends the reading to the microcontroller.

Microcontroller (e.g., Arduino/NodeMCU):

Processes sensor data, runs detection algorithms, and controls the alert system.

GSM Module (e.g., SIM800L):

Sends SMS alerts to predefined contacts when a fall or health issue is detected.

Buzzer:

Emits a sound alarm when a fall or abnormal reading is detected

Display ( e.g., OLED/LCD) [optional]:

Shows real-time heart rate and temperature readings for easy monitoring

Result: The system helps elderly people.

Chapter 3.  CODE DESCRIPTION

The code begins by importing special libraries that help the Arduino work with sensors like the MPU6050 (for movement detection) and GSM module (for sending messages).

The MPU6050 sensor is set up to measure movement and orientation. A GSM module is also initialized using two digital pins to send SMS alerts.

The code checks if the movement (acceleration) values cross a certain level (threshold). If the value is too high, it means a fall might have happened.

The MPU6050 sensor reads values of acceleration (movement) and stores them. This helps detect if a sudden motion has occurred, such as a fall.

The code also reads data from the heart rate sensor and the temperature sensor. If the heart rate or body temperature is too high or too low, it detects a possible health issue.

If a fall or abnormal health condition is detected, the GSM module is used to send a text message to a saved mobile number, informing that an emergency has occurred.

The loop() part of the code runs continuously. It keeps checking for falls and health status every second and takes action if anything unusual is detected.

3.1  ARDUINO CODE

#include <Wire.h>

#include <Adafruit_ADXL345_U.h>

#include <Adafruit_Sensor.h>

#include <LiquidCrystal_I2C.h>

#define BUZZER_PIN D5

#define TEMP_PIN A0

Adafruit_ADXL345_Unified accel = Adafruit_ADXL345_Unified(123);

LiquidCrystal_I2C lcd(0x27, 16, 2);  // Address might be 0x3F on some displays

void setup() {

Serial.begin(115200);

Wire.begin(D2, D1); // SDA, SCL

// LCD

lcd.begin();

lcd.backlight();

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Initializing...");

// ADXL345

if (!accel.begin()) {

lcd.setCursor(0, 1);

lcd.print("ADXL NOT Found!");

while (1);

}

accel.setRange(ADXL345_RANGE_16_G);

// Buzzer

pinMode(BUZZER_PIN, OUTPUT);

delay(1000);

lcd.clear();

}

void loop() {

// TEMP

int analogValue = analogRead(TEMP_PIN);

float voltage = analogValue * 3.3 / 1024.0;

float temperatureC = voltage * 100;

// ADXL345

sensors_event_t event;

accel.getEvent(&event);

float x = abs(event.acceleration.x);

float y = abs(event.acceleration.y);

float z = abs(event.acceleration.z);

bool fallDetected = (x > 12 || y > 12 || z > 12);  // simple threshold

// LCD Output

lcd.setCursor(0, 0);

lcd.print("Temp: ");

lcd.print(temperatureC);

lcd.print(" C   ");

lcd.setCursor(0, 1);

if (fallDetected) {

lcd.print("Fall Detected!  ");

digitalWrite(BUZZER_PIN, HIGH);

} else {

lcd.print("No Fall         ");

digitalWrite(BUZZER_PIN, LOW);

}

delay(500);

}

CHAPTER 4.  RESULTS & DISCUSSION

The development and testing of the Elderly Fall and Health Monitoring Device demonstrated successful real-time monitoring of both movement and vital health parameters. The accelerometer and gyroscope effectively detected sudden changes in motion, accurately identifying simulated falls. The heart rate and temperature sensors provided continuous, reliable readings of the user’s physiological state. When a fall or abnormal health condition was detected, the GSM module successfully sent instant SMS alerts to predefined emergency contacts. This ensured quick response and assistance, even when the elderly individual was alone. Overall, the device proved to be a cost-effective and efficient solution for enhancing the safety and independence of elderly people, with the potential for further improvement through GPS integration and mobile app support.

4.1 PROTOTYPE

Fig 4.1

The working circuit

CHAPTER 5.  CONCLUSION , APPLICATIONS & FUTURE SCOPE

CONCLUSION:

The Elderly Fall and Health Monitoring Device is a reliable and affordable solution that helps ensure the safety of elderly individuals by detecting falls and monitoring vital signs in real time, while providing immediate alerts to caregivers for timely assistance.

APPPLICATION:

- Home Care Monitoring:

Helps families monitor elderly members living alone by providing real-time alerts during emergencies.

- Hospitals and Elderly Care Centers:

Used in hospitals, old-age homes, or rehabilitation centers to continuously monitor patients' movements and health.

- Remote Health Monitoring:

Enables doctors or caregivers to track vital signs remotely, reducing the need for frequent hospital visits

FUTURE SCOPE:

- Integration with Mobile Apps:

Development of a dedicated mobile application to track real-time data and history of falls and health parameters.

- GPS-Based Location Tracking:

Adding GPS functionality to send the exact location of the user during emergencies for faster help.

- Voice Alert and SOS Button:

Including a voice alert system or panic/SOS button for manual emergency activation.

- Advanced AI and Machine Learning:

Using AI to improve fall detection accuracy and predict possible health risks based on sensor data patterns.

CHAPTER 6.  REFERENCES

- World Health Organization (WHO) – “Falls”
https://www.who.int/news-room/fact-sheets/detail/falls
(Provides global statistics and impact of falls among the elderly)

- IEEE Xplore Digital Library – Research papers on fall detection systems
https://ieeexplore.ieee.org/
(Search keywords like “elderly fall detection”, “health monitoring using IoT”)

- Arduino Official Website – Documentation for hardware and coding
https://www.arduino.cc/
(Useful for understanding how to use Arduino boards and sensors)

CHAPTER 7.  APPENDICES

Appendix 1: Component Datasheets

Appendix 2: Circuit Diagram

Appendix 3: Arduino Code